2D Barcode Scanner

User Manual

1、Product Introduction	6

1.1 Factory default	6

1.2 Get device information	6

2 Wireless settings	6

2.1 Pairing setup code	6

2.2 BLE Bluetooth Mode	7

2.3 Mode Selection	7

2.4 Set the upload data speed delay	8

2.5 Check software version	8

2.6 Set sleep time	9

2.7 USB KBW	9

2.8 USB COM	9

2.10 Output forced letter case conversion	11

2.11 Output format	11

2.12 GS Seting	12

2.13 Battery Query	12

3 Input/Output Settings	12

3.1 Startup Beeper	12

3.2 Trigger Click Beeper	13

3.3 Good Read Beeper	13

3.3.1 Good Read Beeper Volume	13

3.3.2 Good Read Beeper Frequency	13

3.3.3 Good Read Beeper Duration	14

3.3.4 Error Read Beeper Frequency	14

3.3.5 Good Read LED	14

3.4 Manual mode	15

3.5 Induction mode	15

3.5.1 sensitivity	15

3.6 Continuous mode	16

4 Data Editing	17

4.1 Add Preﬁx or Suﬃx	17

4.12 Suﬃx Example	17

4.13 To Add a Suﬃx to All Symbologies	18

4.2 Function Code Transmit	18

5 Barcode symbol parameters	19

5.1 Global operations	19

5.1.1 Operations on all symbol types	19

5.1.2 Operation on all one-dimensional bar code symbol types	19

5.1.3 Operation of all 2D barcode symbol types	19

5.2 Inverse color code setting	20

5.2.1 Operate all reverse color codes	20

5.3 Codabar	20

5.3.1 Enable/Disable Codabar	20

5.3.2 Start/Stop Characters	20

5.3.3 Check Character	21

5.4 Code 39	22

5.4.1 Enable/Disable Code 39	22

5.4.2 Code 39 Start/ Stop Characters	22

5.4.3 Code 39 Message Length	23

5.4.4 Code 39 Check and output check	23

5.4.5 Code 32 Pharmaceutical (PARAF)	23

5.4.6 Code 39 Full ASCII Support	24

5.5 Interleaved 2 of 5	24

5.5.1 Enable /Disable InterLeaved25	24

5.5.2 InterLeaved25 Message Length	24

5.5.3 InterLeaved25 Check and output check	25

5.6 NEC 2 of 5	25

5.6.1 Enable /Disable NEC 2 of 5	25

5.6.2 NEC 2 of 5 Message Length	25

5.6.3 NEC 2 of 5 Check and output check	26

5.7 Code 93	26

5.7.1 Enable/Disable Code 93	26

5.7.2 Code 93 Message Length	26

5.8 Industrial 2 of 5	27

5.8.1 Enable /Disable Industrial 25	27

5.8.2 Industrial 2 of 5 Message Length	27

5.9 Straight 2 of 5 IATA (two-bar start/stop)	27

5.9.1 Enable /Disable Straight 2 of 5 IATA	27

5.9.2 Straight 2 of 5 IATA Message Length	28

5.10 Matrix 2 of 5	28

5.10.1 Enable/Disable Matrix 25	28

5.10.2 Matrix 25 Message Length	28

5.10.3 Check and output check	29

5.11 Code 11	29

5.11.1 Enable/Disable Code 11	29

5.11.2 Code 11 Message Length	29

5.12 Code 128	29

5.12.1 Enable/Disable Code 128	29

5.12.2 ISBT 128 Concatenation	30

5.12.3 Code 128 Message Length	30

5.13 GS1 128	30

5.13.1 Enable/Disable GS1 128	30

5.13.2 GS1 128 Message Length	31

5.14 UPCA	31

5.14.1 Enable/Disable UPCA	31

5.14.2 UPCA Output verification	31

5.14.3 UPCA Output system characters	31

5.14.4 UPCA Extension code	32

5.14.5 UPCA Addenda Required	32

5.14.6 UPCA Addenda Separator	32

5.14.7 UPC-A to EAN-13	33

5.15 UPCE0	33

5.15.1 Enable /Disable UPC-E0	33

5.15.2 Output verification	33

5.15.3 Output system characters	33

5.15.4 UPCE0 Extension code	34

5.15.5 UPCE0 Addenda Required	34

5.15.6 UPCE0 Addenda Separator	34

5.16 UPCE1	35

5.16.1 Enable /Disable UPC-E1	35

5.17 EAN-13	35

5.17.1 Enable/Disable  EAN-13	35

5.17.2 EAN 13 Output verification	35

5.17.3 Extension code	35

5.17.4 EAN13 to ISBN	36

5.17.5 EAN13 Addenda Required	36

5.17.6 EAN13 Addenda Separator	36

5.18 EAN-8	37

5.18.1 Enable/Disable EAN-8	37

5.18.2 Output verification	37

5.18.3 Extension code	37

5.18.4 EAN8 Addenda Required	38

5.18.5 EAN8 Addenda Separator	38

5.19 MSI Plessey	38

5.19.1 Enable/Disable MSI Plessey	38

5.19.2 Message Length	38

5.20 GS1 DataBar Omnidirectional	39

5.21 GS1 DataBar Limited	39

5.22 GS1 DataBar Expanded	39

5.23 PDF 417	39

5.24 QR Code	40

5.25 Data Matrix	40

5.26 Aztec Code	40

5.27 China Post (Hong Kong 2 of 5)	40

5.28 Korea Post	40

5.29 Han Xin Code	41

5.30 Maxi Code	41

5.31 Micro pdf417	41

5.32 Gs1 Composite Codes	41

5.32 Codablock A	41

5.33 Disable GS1 Data Matrix	42

6 Data Code	42

6.1 Data code 0~F	42

6.2 Save	43

Appendix A：ASCII Code Table	44

- 1、Product Introduction

  - Factory default

- End：Enter(\r)。

Factory default

  - Get device information

- Obtain the version number

- 2 Wireless settings

  - 2.1 Pairing setup code

- ①Pair the receiver: Scan the following two barcodes in sequence.And connect the receiver to computer；

| 2.4G Mode | Connect |
| --- | --- |

②Pair Bluetooth devices in HID mode: Scan the following two barcodes in sequence, turn on the Bluetooth device to search for RB_Scanner_HID connection.

| HIDMode | Connect |
| --- | --- |

  - 2.2  BLE Bluetooth Mode

Pairing Bluetooth devices: Scan the following two barcodes in sequence, and perform Bluetooth search RB_Scanner_BLE on the software that supports BLE

Connect.Note: BLE mode needs to be output on specific software (Bluetooth serial port)

| BLE Mode | Connect |
| --- | --- |

  - 2.3 Mode Selection

| Instant upload mode | Inventory Mode |
| --- | --- |
| *Over-the-distance storage mode |  |

①Operations in Inventory Mode

| Upload all data | Upload new data (that is, data that has not been uploaded before) |
| --- | --- |
| Display saved data | Showing unuploaded data |
| Clear all data |  |

  - 2.4 Set the upload data speed delay

| *No delay | Delay 10ms |
| --- | --- |
| Delay 20ms |  |

  - 2.5 Check software version

| Check the scanner software version | Check the receiving end version number |
| --- | --- |

  - 2.6 Set sleep time

| No sleep | *1min |
| --- | --- |
| 2min | 5min |
| 10min |  |

  - 2.7 USB KBW

When the scanner is connected to the host using a USB receiver, the scanner can be configured as a standard keyboard input by scanning the USB KBW setup code.

*USB KBW keyboard

  - 2.8 USB COM

When the scanner is connected to the host using a USB receiver, the receiver virtual serial port output mode can be changed by scanning the USB COM setup code.

USB COM

2.9 multi-language keyboards

When the scanner recognizes it as a keyboard input device, different countries have different input characters, so you need to set different national languages. The keyboard defaults to the US language.

| *U.S | Belgium (French) |
| --- | --- |
| Brazil (Portuguese) | Canada |
| Czech | Denmark |
| Finland | France |
| Germany | Italy |
| Portugal | Spain |
| Türkiye-F | Türkiye-Q |
| U.K. | Japan |
| Russia | Vietnamese |

  - 2.10 Output forced letter case conversion

Letter conversion, when outputting barcodes with letter content, you can configure the output result to be all uppercase or all lowercase. For example, if the barcode content is: ab123dE, if you scan the "Convert to uppercase" barcode, the output result is: AB123DE; if you scan the "Convert to lowercase" barcode, the output result is: abc123de; the default case is not converted.

| *Do not convert | Uppercase |
| --- | --- |
| Lowercase | Case conversion |

  - 2.11 Output format

| *UTF-8 | GBK |
| --- | --- |

  - 2.12 GS Seting

| *Eanble GS | Disable GS |
| --- | --- |

- 2.13 Battery Query

After scanning the setup code, the battery percentage will be output

- 3 Input/Output Settings

This chapter mainly introduces the conﬁguration of the beep and LED of the barcode scanner when it is powered on, decoded, and triggered by the button.

  - 3.1 Startup Beeper

The scanner can be programmed to beep when it’s started up. Default = Startup Beeper On.

| Startup Beeper Off | * Startup Beeper On |
| --- | --- |

  - 3.2  Trigger Click Beeper

To hear an audible click every time the scanner button is pressed, scan the Trigger Click Beeper On barcode below. Default = Trigger Click Beeper Off.

| *Trigger Click Beeper Off | Trigger Click Beeper On |
| --- | --- |

    - 3.3  Good Read Beeper

The beeper may be programmed On or Off in response to a good read. Default = Good Read Beeper On.

| Good Read Beeper Off | * Good Read Beeper On |
| --- | --- |

- 3.3.1  Good Read Beeper Volume

The beeper volume codes modify the volume of the beep the scanner emits on a good read. Default = High.

| Low | Medium |
| --- | --- |
| * High | Off |

- 3.3.2 Good Read Beeper Frequency

The beeper frequency codes modify the frequency of the beep the scanner emits on a good read. Default = Medium.

| Low (1600 Hz) | * Medium (2400 Hz) |
| --- | --- |
| High (4200 Hz) |  |

- 3.3.3  Good Read Beeper Duration

The beeper duration codes modify the length of the beep the scanner emits on a good read. Default = Normal .

| * Normal | Short |
| --- | --- |

    - 3.3.4  Error Read Beeper Frequency

The beeper frequency codes modify the frequency of the sound the scan- ner emits when there is a bad read or error. Default = Razz.

| * Razz (250 Hz) | Medium (3250 Hz) |
| --- | --- |
| High (4200 Hz) |  |

    - 3.3.5  Good Read LED

The LED indicator can be programmed On or Off in response to a good read. Default = On.

| * Good Read LED On | Good Read LED Off |
| --- | --- |

- 3.4  Manual mode

The manual reading mode is the default reading mode. In this mode, the recognition engine starts reading the code after the user presses the trigger key, and stops reading the code after successfully outputting information or the user releases the trigger key.

*Manual mode

- 3.5 Induction mode

After setting up, there is no need to trigger, and the recognition engine immediately starts monitoring the brightness of the surrounding environment. When the scene changes, the recognition engine waits for the set stabilization time to end before starting to read the code. If the following situation does not occur, the reading engine will cycle in the above way: if the barcode is not scanned within a single reading time, the reading engine will automatically pause reading and enter the monitoring state. In the induction reading mode, the reading engine can also start reading the code after the user presses the trigger button. When the code reading is successful and outputs information or the user releases the trigger button, it will switch to induction mode for 5 seconds and continue to monitor the brightness of the surrounding environment.

Induction mode

- 3.5.1  sensitivity

Self sensing sensitivity, the higher the sensitivity, the easier it is to wake up, but it is easy to accidentally wake up the device to decoding mode, default to high.

| High* | Mid |
| --- | --- |
| Low |  |

- 3.6  Continuous mode

After setting up, there is no need to trigger. The reading engine immediately starts reading the code. During the reading process, the user can also manually pause the reading by clicking the trigger button. When the reading is successful and outputs information or the user releases the trigger button, it will switch to continuous mode for 5 seconds

Continuous mode

#### 3.61 Same barcode reading delay

Sensing mode and continuous mode scanning can be used to set the time interval between barcode scanning equipment reading the next barcode after reading one barcode。

| No time interval | *500MS |
| --- | --- |
| 750MS | 1000MS |
| 1500MS |  |

  - 4  Data Editing

  - 4.1  Add Preﬁx or Suﬃx

Step 1. Scan the Add Preﬁx or Add Suﬃx symbol

Step 2. Determine the 2 digit Hex value from the Symbology Chart for the symbology to 			which you want to apply the preﬁx or suﬃx. For example, for Code 11, Code ID is “h” 		and Hex ID is “68”.

Step 3. Scan the 2 hex digits from the Programming Chart inside the back cover of this manual or scan 9, 9 for all symbologies.

Step 4. Determine the hex value from the ASCII Conversion Chart , for the preﬁx or suﬃx you wish to enter.

Step 5. Scan the 2 digit hex value from the Programming Chart inside the back cover of this manual.

Step 6. Repeat Steps 4 and 5 for every preﬁx or suﬃx character.

Step 7. Scan Save to exit and save, or scan Discard to exit without saving.

| Add Suﬃx | Add Suﬃx |
| --- | --- |

- 4.12 Suﬃx Example

Add a Suﬃx to a speciﬁc symbology

To send a CR (carriage return)Suﬃx for code 128. only:

Step 1. Scan Add Suﬃx.

Step 2. Determine the 2 digit hex value from the Symbology Charts for code 128.

Step 3. Scan 6, 3 from the Programming Chart inside the back cover of this manual.

Step 4. Determine the hex value from the ASCII Conversion Chart , for the CR (carriage return).

Step 5. Scan 0, D from the Programming Chart inside the back cover of this manual.

Step 6. Scan Save, or scan Discard to exit without saving.

| Step 1 | Step 2 |
| --- | --- |
| Add Suﬃx | 6 |
| Step 3 | Step 4 |
| A | 0 |
| Step 5 | Step 6 |
| D | Save |

    - 4.13 To Add a Suﬃx to All Symbologies

| *Add CR Suﬃx All Symbologies | Add LF Suﬃx All Symbologies |
| --- | --- |
| Add CR and LF Suﬃx All Symbologies | Add tab Suﬃx All Symbologies |

    - 4.2 Function Code Transmit

When this selection is enabled and function codes are contained within the scanned data, the scanner transmits the function code to the terminal. Default = Disable

| Enable | *Disable |
| --- | --- |

- 5  Barcode symbol parameters

  - 5.1  Global operations

    - 5.1.1  Operations on all symbol types

Reading the following setting codes will operate on all supported symbol types, allowing or prohibiting reading. After prohibiting reading all types, only setting codes are allowed to be read.

| Allow reading all types | Prohibit reading all types |
| --- | --- |

    - 5.1.2  Operation on all one-dimensional bar code symbol types

Read the following setting codes, and only perform unified operations on all one-dimensional bar code symbol types, or all allow reading, or all prohibit reading.

| Allows to read all one-dimensional barcode types | Prohibit reading all 1D barcode type |
| --- | --- |

    - 5.1.3  Operation of all 2D barcode symbol types

Read the following setting codes, and only perform unified operations on all 2D barcode symbol types, or all allow reading, or all prohibit reading.

| Allow to read all 2D barcode types | Prohibit reading all QR code types |
| --- | --- |

  - 5.2   Inverse color code setting

If this configuration is turned on, the recognition speed will be affected. Please turn it on in the scene you need to use.

    - 5.2.1 Operate all reverse color codes

| Allow to read the reverse color code | *Prohibit read the reverse color code |
| --- | --- |

  - 5.3  Codabar

    - 5.3.1 Enable/Disable Codabar

| Enable Codabar | Disable Codabar |
| --- | --- |

    - 5.3.2 Start/Stop Characters

Start/Stop characters identify the leading and trailing ends of the barcode. You may either transmit, or not transmit Start/Stop characters.Default = Don’t Transmit.

| Transmit | * Don’t Transmit |
| --- | --- |

  - 5.3.3 Check Character

No Check Character indicates that the scanner reads and transmits barcode data with or without a check character.

When Check Character is set to Validate and Transmit, the scanner will only read Codabar barcodes printed with a check character, and will transmit this character at the end of the scanned data.

When Check Character is set to Validate, but Don’t Transmit, the unit will only read Codabar barcodes printed with a check character, but will not transmit the check character with the scanned data. Default = No Check Character.

| * No Check Character | Validate but Don’t Transmit |
| --- | --- |
| Validate and Transmit |  |

  - 5.3.4 Concatenation

Codabar supports symbol concatenation. When you enable concatena- tion, the scanner looks for a Codabar symbol having a “D” start character, adjacent to a symbol having a “D” stop character. In this case the two mes- sages are concatenated into one with the “D” characters omitted.

A 1   2 3 4 D	D 5 6 7 8A

Select Require to prevent the scanner from decoding a single “D” Codabar symbol without its 	companion. This selection has no effect on Codabar symbols without Stop/Start D characters.

| On | * Off |
| --- | --- |
| Require |  |

  - 5.3.5 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 2-60. Minimum Default = 4, Maximum Default = 60.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.4 Code 39

    - 5.4.1 Enable/Disable Code 39

| *Enable Code 39 | Disable Code 39 |
| --- | --- |

    - 5.4.2  Code 39 Start/ Stop Characters

Start/Stop characters identify the leading and trailing ends of the barcode. You may either transmit, or not transmit Start/Stop characters. Default = Don’t Transmit.

| Transmit | * Don’t Transmit |
| --- | --- |

    - 5.4.3 Code 39 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths

= 0-48. Minimum Default = 0, Maximum Default = 48.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

    - 5.4.4 Code 39 Check and output check

No Check Character indicates that the scanner reads and transmits bar- code data with or without a check character.

When Check Character is set to Validate, but Don’t Transmit, the unit only reads Code 39 barcodes printed with a check character, but will not transmit the check character with the scanned data.

When Check Character is set to Validate and Transmit, the scanner only reads Code 39 barcodes printed with a check character, and will transmit this character at the end of the scanned data. Default = No Check Character.

| * No Check Character |  |
| --- | --- |
| Validate, but Don’t Transmit | Validate and Transmit |

    - 5.4.5 Code 32 Pharmaceutical (PARAF)

Code 32 Pharmaceutical is a form of the Code 39 symbology used by Ital- ian pharmacies. This symbology is also known as PARAF.

When you conﬁgure code32, you need to turn on code39 before you conﬁgure it.

| *Disable Code32 | Enable Code32 |
| --- | --- |

    - 5.4.6 Code 39 Full ASCII Support

The encoding method of Code 39 can include the representation of all ASCII characters. By setting, the reading module can support bar codes containing the full ASCII character set.

| Enable Full ASCII | *Disable Full ASCII |
| --- | --- |

  - 5.5   Interleaved 2 of 5

    - 5.5.1 Enable /Disable InterLeaved25

| *Enable InterLeaved25 | Disable InterLeaved25 |
| --- | --- |

    - 5.5.2 InterLeaved25 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths= 2-80. Minimum Default = 4, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

    - 5.5.3 InterLeaved25 Check and output check

No Check Digit indicates that the scanner reads and transmits barcode data with or without a check digit.

When Check Digit is set to Validate, but Don’t Transmit, the unit only reads Interleaved 2 of 5 barcodes printed with a check digit, but will not transmit the check digit with the scanned data.

When Check Digit is set to Validate and Transmit, the scanner only reads Interleaved 2 of 5 barcodes printed with a check digit, and will transmit this digit at the end of the scanned data. Default = No Check Digit.

| * No Check Character |  |
| --- | --- |
| Validate, but Don’t Transmit | Validate and Transmit |

  - 5.6   NEC 2 of 5

    - 5.6.1 Enable /Disable NEC 2 of 5

| *Enable InterLeaved25 | Disable InterLeaved25 |
| --- | --- |

    - 5.6.2 NEC 2 of 5 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths= 2-80. Minimum Default = 4, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

    - 5.6.3 NEC 2 of 5 Check and output check

No Check Digit indicates that the scanner reads and transmits barcode data with or without a check digit.

When Check Digit is set to Validate, but Don’t Transmit, the unit only reads NEC 2 of 5 barcodes printed with a check digit, but will not transmit the check digit with the scanned data.

When Check Digit is set to Validate and Transmit, the scanner only reads NEC 2 of 5 barcodes printed with a check digit, and will transmit this digit at the end of the scanned data. Default = No Check Digit

| * No Check Character |  |
| --- | --- |
| Validate, but Don’t Transmit | Validate and Transmit |

  - 5.7   Code 93

    - 5.7.1  Enable/Disable Code 93

| *Enable Code 93 | Disable Code 93 |
| --- | --- |

    - 5.7.2  Code 93 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 0-80. Minimum Default = 0, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.8   Industrial 2 of 5

    - 5.8.1  Enable /Disable Industrial 25

| Enable Industrial 25 | *Disable Industrial 25 |
| --- | --- |

    - 5.8.2 Industrial 2 of 5 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-48. Minimum Default = 4, Maximum Default = 48.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.9   Straight 2 of 5 IATA (two-bar start/stop)

    - 5.9.1  Enable /Disable Straight 2 of 5 IATA

| Enable Straight 2 of 5 IATA | *Disable Straight 2 of 5 IATA |
| --- | --- |

    - 5.9.2 Straight 2 of 5 IATA Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-48. Minimum Default = 4, Maximum Default = 48.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.10  Matrix 2 of 5

    - 5.10.1 Enable/Disable Matrix 25

| Enable Matrix 25 | *Disable Matrix 25 |
| --- | --- |

    - 5.10.2 Matrix 25 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 4, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

    - 5.10.3  Check and output check

| Enable Check Function | Disable Check Function |
| --- | --- |

  - 5.11  Code 11

    - 5.11.1  Enable/Disable Code 11

| Enable Code 11 | *Disable Code11 |
| --- | --- |

    - 5.11.2 Code 11 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 4, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.12  Code 128

    - 5.12.1 Enable/Disable Code 128

| *Enable Code 128 | Disable Code 128 |
| --- | --- |

    - 5.12.2 ISBT 128 Concatenation

| ISBT 128 On | *ISBT 128 Off |
| --- | --- |

    - 5.12.3 Code 128 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default =1, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.13  GS1 128

    - 5.13.1 Enable/Disable GS1 128

| *Enable GS1 128 | Disable GS1 128 |
| --- | --- |

    - 5.13.2 GS1 128 Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 1, Maximum Default = 80.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.14   UPCA

    - 5.14.1  Enable/Disable UPCA

| *Enable UPCA | Disable UPCA |
| --- | --- |

    - 5.14.2  UPCA Output verification

| *Output check | No output check |
| --- | --- |

    - 5.14.3 UPCA Output system characters

| *Output system characters | No output system characters |
| --- | --- |

    - 5.14.4 UPCA Extension code

After setting to "Read 2-digit extension code" or "Read 5-digit extension code", the reader module can read barcode symbols with extension codes, or barcode symbols without extension codes. After setting to "Do not read 2-digit extension code" or "Do not read 5-digit extension code", the extension code attached to the bar code symbol will not be read and output.

| *Disable 2-digit extension code | Enable 2-digit extension code |
| --- | --- |
| *Disable 5-digit extension code | Enable 5-digit extension code |

    - 5.14.5 UPCA Addenda Required

When Required is scanned, the scanner will only read UPC-A barcodes that have addenda. You must then turn on a 2 or 5 digit addenda. Default = Not Required.

| Required | * Not Required |
| --- | --- |

    - 5.14.6 UPCA Addenda Separator

When this feature is on, there is a space between the data from the barcode and the data from the addenda. When turned off, there is no space. Default= On.

| *On | Off |
| --- | --- |

    - 5.14.7 UPC-A to EAN-13

| On | *Off |
| --- | --- |

  - 5.15   UPCE0

    - 5.15.1  Enable /Disable UPC-E0

| *Enable UPCE0 | Disable UPCE0 |
| --- | --- |

    - 5.15.2  Output verification

| *Output check | No Output check |
| --- | --- |

    - 5.15.3  Output system characters

| *Output system characters | No output system characters |
| --- | --- |

    - 5.15.4 UPCE0 Extension code

After setting to "Read 2-digit extension code" or "Read 5-digit extension code", the reader module can read barcode symbols with extension codes, or barcode symbols without extension codes. After setting to "Do not read 2-digit extension code" or "Do not read 5-digit extension code", the extension code attached to the bar code symbol will not be read and output.

| *Disable 2-digit extension code | Enable 2-digit extension code |
| --- | --- |
| *Disable 5-digit extension code | Enable 5-digit extension code |

    - 5.15.5 UPCE0 Addenda Required

When Required is scanned, the scanner will only read UPC-A barcodes that have addenda. You must then turn on a 2 or 5 digit addenda. Default = Not Required.

| Required | * Not Required |
| --- | --- |

    - 5.15.6 UPCE0 Addenda Separator

When this feature is on, there is a space between the data from the barcode and the data from the addenda. When turned off, there is no space. Default= On.

| *On | Off |
| --- | --- |

  - 5.16   UPCE1

    - 5.16.1  Enable /Disable UPC-E1

| *Enable UPCE1 | Disable UPCE1 |
| --- | --- |

  - 5.17   EAN-13

    - 5.17.1 Enable/Disable  EAN-13

| *Enable EAN-13 | Disable EAN-13 |
| --- | --- |

    - 5.17.2 EAN 13 Output verification

| *Output check | No Output check |
| --- | --- |

    - 5.17.3 Extension code

After setting to "Read 2-digit extension code" or "Read 5-digit extension code", the reader module can read barcode symbols with extension codes, or barcode symbols without extension codes. After setting to "Do not read 2-digit extension code" or "Do not read 5-digit extension code", the extension code attached to the bar code symbol will not be read and output.

| *Disable 2-digit extension code | Enable 2-digit extension code |
| --- | --- |
| *Disable 5-digit extension code | Enable 5-digit extension code |

    - 5.17.4 EAN13 to ISBN

Other configurations are the same as EAN13.

| *Disable EAN13 to ISSN | Enable EAN13 to ISSN |
| --- | --- |

    - 5.17.5 EAN13 Addenda Required

When Required is scanned, the scanner will only read UPC-A barcodes that have addenda. You must then turn on a 2 or 5 digit addenda. Default = Not Required.

| Required | * Not Required |
| --- | --- |

    - 5.17.6 EAN13 Addenda Separator

When this feature is on, there is a space between the data from the barcode and the data from the addenda. When turned off, there is no space. Default= On.

| *On | Off |
| --- | --- |

  - 5.18    EAN-8

    - 5.18.1 Enable/Disable EAN-8

| Enable EAN-8 | *Disable EAN-8 |
| --- | --- |

    - 5.18.2 Output verification

EAN-8 The barcode data is fixed to 8 bytes, of which the last byte is the checksum.

| *Output check | No output check |
| --- | --- |

    - 5.18.3 Extension code

After setting to "Read 2-digit extension code" or "Read 5-digit extension code", the reader module can read barcode symbols with extension codes, or barcode symbols without extension codes. After setting to "Do not read 2-digit extension code" or "Do not read 5-digit extension code", the extension code attached to the bar code symbol will not be read and output.

| *Disable 2-digit extension code | Enable 2-digit extension code |
| --- | --- |
| *Disable 5-digit extension code | Enable 5-digit extension code |

    - 5.18.4 EAN8 Addenda Required

When Required is scanned, the scanner will only read UPC-A barcodes that have addenda. You must then turn on a 2 or 5 digit addenda. Default = Not Required.

| Required | * Not Required |
| --- | --- |

    - 5.18.5 EAN8 Addenda Separator

When this feature is on, there is a space between the data from the barcode and the data from the addenda. When turned off, there is no space. Default= On.

| *On | Off |
| --- | --- |

  - 5.19  MSI Plessey

    - 5.19.1   Enable/Disable MSI Plessey

| Enable MSI Plessey | *Disable MSI Plessey |
| --- | --- |

    - 5.19.2  Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additionalinformation. Minimum and Maximum lengths = 4-48. Minimum Default = 4, Maximum Default = 48.

| Minimum Message Length | Maximum Message Length |
| --- | --- |

  - 5.20 GS1 DataBar Omnidirectional

| *Enable GS1 DataBar Omnidirectional | Disable GS1 DataBar Omnidirectional |
| --- | --- |

  - 5.21 GS1 DataBar Limited

| *Enable GS1 DataBar Limited | Disable GS1 DataBar Limited |
| --- | --- |

  - 5.22  GS1 DataBar Expanded

| *Enable GS1 DataBar Expanded | Disable GS1 DataBar Expanded |
| --- | --- |

  - 5.23 PDF 417

| *Enable PDF 417 | Disable PDF 417 |
| --- | --- |

  - 5.24  QR Code

| *Enable QR | Disable QR |
| --- | --- |

  - 525  Data Matrix

| *Enable Data Matrix | Disable Data Matrix |
| --- | --- |

  - 5.26  Aztec Code

| Enable Aztec Code | *Disable Aztec Code |
| --- | --- |

  - 5.27  China Post (Hong Kong 2 of 5)

| Enable China Post (Hong Kong 2 of 5) | *Disable China Post (Hong Kong 2 of 5) |
| --- | --- |

  - 5.28  Korea Post

| Enable Korea Post | *Disable Korea Post |
| --- | --- |

  - 5.29  Han Xin Code

| Enable Han Xin Code | *Disable Han Xin Code |
| --- | --- |

  - 5.30  Maxi Code

| Enable Maxi Code | *Disable Maxi Code |
| --- | --- |

  - 5.31  Micro pdf417

| *Enable Micro pdf417 | Disable Micro pdf417 |
| --- | --- |

  - 5.32  Gs1 Composite Codes

| Enable Gs1 Composite Codes | *Disable Gs1 Composite Codes |
| --- | --- |

  - 5.32  Codablock A

| Enable Codablock A | *Disable Codablock A |
| --- | --- |

  - 5.33  Disable GS1 Data Matrix

| Enable GS1 Data Matrix | *Disable GS1 Data Matrix |
| --- | --- |

- 6  Data Code

  - 6.1  Data code 0~F

| Data Code0 | Data Code1 |
| --- | --- |
| Data Code2 | Data Code3 |
| Data Code4 | Data Code5 |
| Data Code6 | Data Code7 |
| Data Code8 | Data Code9 |
| Data CodeA | Data CodeB |
| Data CodeC | Data CodeD |
| Data CodeE | Data CodeF |

  - 6.2  Save

| Save | Discard |
| --- | --- |

Note: If an error occurs while scanning a letter or number (before scanning the "Save" barcode), scan the "Discard" barcode, rescan the correct letter or number, and then scan the "Save" barcode.

- Appendix A：ASCII Code Table

| Hexadecimal | Decimal | Character |
| --- | --- | --- |
| 00 | 0 | NUL (Nullchar.) |
| 01 | 1 | SOH (StartofHeader) |
| 02 | 2 | STX (Start ofText) |
| 03 | 3 | ETX (End ofText) |
| 04 | 4 | EOT (EndofTransmission) |
| 05 | 5 | ENQ (Enquiry) |
| 06 | 6 | ACK (Acknowledgment) |
| 07 | 7 | BEL (Bell) |
| 08 | 8 | BS (Backspace) |
| 09 | 9 | HT (HorizontalTab) |
| 0a | 10 | LF (LineFeed) |
| 0b | 11 | VT (VerticalTab) |
| 0c | 12 | FF (FormFeed) |
| 0d | 13 | CR (CarriageReturn) |
| 0e | 14 | SO (ShiftOut) |
| 0f | 15 | SI (ShiftIn) |
| 10 | 16 | DLE (DataLinkEscape) |
| 11 | 17 | DC1 (XON)(DeviceControl1) |
| 12 | 18 | DC2 (DeviceControl2) |
| 13 | 19 | DC3 (XOFF)(DeviceControl3) |
| 14 | 20 | DC4 (DeviceControl4) |
| 15 | 21 | NAK (NegativeAcknowledgment) |
| 16 | 22 | SYN (SynchronousIdle) |
| 17 | 23 | ETB (EndofTrans.Block) |
| 18 | 24 | CAN (Cancel) |
| 19 | 25 | EM (EndofMedium) |
| 1a | 26 | SUB (Substitute) |
| 1b | 27 | ESC (Escape) |
| 1c | 28 | FS (FileSeparator) |
| 1d | 29 | GS (GroupSeparator) |
| 1e | 30 | RS (Request toSend) |
| 1f | 31 | US (UnitSeparator) |
| 20 | 32 | SP (Space) |
| 21 | 33 | ! (ExclamationMark) |
| 22 | 34 | " (DoubleQuote) |
| 23 | 35 | # (NumberSign) |
| 24 | 36 | $ (DollarSign) |
| Hexadecimal | Decimal | Character |
| 25 | 37 | % (Percent) |
| 26 | 38 | & (Ampersand) |
| 27 | 39 | ` (SingleQuote) |
| 28 | 40 | ( (Right/ClosingParenthesis) |
| 29 | 41 | ) (Right/ClosingParenthesis) |
| 2a | 42 | * (Asterisk) |
| 2b | 43 | + (Plus) |
| 2c | 44 | , (Comma) |
| 2d | 45 | - (Minus/Dash) |
| 2e | 46 | . (Dot) |
| 2f | 47 | / (ForwardSlash) |
| 30 | 48 | 0 |
| 31 | 49 | 1 |
| 32 | 50 | 2 |
| 33 | 51 | 3 |
| 34 | 52 | 4 |
| 35 | 53 | 5 |
| 36 | 54 | 6 |
| 37 | 55 | 7 |
| 38 | 56 | 8 |
| 39 | 57 | 9 |
| 3a | 58 | : (Colon) |
| 3b | 59 | ; (Semi-colon) |
| 3c | 60 | < (LessThan) |
| 3d | 61 | = (EqualSign) |
| 3e | 62 | > (GreaterThan) |
| 3f | 63 | ? (QuestionMark) |
| 40 | 64 | @ (ATSymbol) |
| 41 | 65 | A |
| 42 | 66 | B |
| 43 | 67 | C |
| 44 | 68 | D |
| 45 | 69 | E |
| 46 | 70 | F |
| 47 | 71 | G |
| 48 | 72 | H |
| 49 | 73 | I |
| 4a | 74 | J |
| 4b | 75 | K |
| 4c | 76 | L |
| Hexadecimal | Decimal | Character |
| 4d | 77 | M |
| 4e | 78 | N |
| 4f | 79 | O |
| 50 | 80 | P |
| 51 | 81 | Q |
| 52 | 82 | R |
| 53 | 83 | S |
| 54 | 84 | T |
| 55 | 85 | U |
| 56 | 86 | V |
| 57 | 87 | W |
| 58 | 88 | X |
| 59 | 89 | Y |
| 5a | 90 | Z |
| 5b | 91 | [ (Left/OpeningBracket) |
| 5c | 92 | \ (BackSlash) |
| 5d | 93 | ] (Right/ClosingBracket) |
| 5e | 94 | ^ (Caret/Circumﬂex) |
| 5f | 95 | _ (Underscore) |
| 60 | 96 | ' (GraveAccent) |
| 61 | 97 | a |
| 62 | 98 | b |
| 63 | 99 | c |
| 64 | 100 | d |
| 65 | 101 | e |
| 66 | 102 | f |
| 67 | 103 | g |
| 68 | 104 | h |
| 69 | 105 | i |
| 6a | 106 | j |
| 6b | 107 | k |
| 6c | 108 | l |
| 6d | 109 | m |
| 6e | 110 | n |
| 6f | 111 | o |
| 70 | 112 | p |
| 71 | 113 | q |
| 72 | 114 | r |
| 73 | 115 | s |
| 74 | 116 | t |
| Hexadecimal | Decimal | Character |
| 75 | 117 | u |
| 76 | 118 | v |
| 77 | 119 | w |
| 78 | 120 | x |
| 79 | 121 | y |
| 7a | 122 | z |
| 7b | 123 | { (Left/OpeningBrace) |
| 7c | 124 | ¦ (VerticalBar) |
| 7d | 125 | } (Right/ClosingBrace) |
| 7e | 126 | ~ (Tilde) |
| 7f | 127 | DEL (Delete) |