catalogue

一 Product introduction	6

1.1 Factory Default	6

1.2 Obtain device information	6

二 Wireless settings	7

2.1 Pairing setup code	7

2.2 BLE BLUETOOTH MODE	7

2.3 Mode Selection	7

2.4 Set the upload data speed delay	8

2.5 Check software version	8

2.6 Set sleep time	9

2.7 USB KBW	9

2.8 USB COM keyboard	9

2.9 Output format	10

2.10 Output forced letter case conversion	11

三、Scanning module settings	12

1.2 Enable/Disable All Symbologies	12

1.3 Video Reverse	12

1.3.1 Trigger Mode	13

1.3.2 Decode Session Timeout	14

1.4 Continuous Mode	15

1.4.1 Mode Entry	15

1.4.2 Decode Session Timeout	15

1.4.3 Timeout Between Decodes	16

1.4.4 Enable/Disable Timeout Between Decodes (Same Barcode)	17

1.4.5 Timeout Between Decodes (Same Barcode)	17

1.5 Sense Mode	18

1.5.1 Mode Entry	18

1.5.2 Sensitivity	18

1.5.3 Image Stabilization Timeout	19

1.5.4 Decode Session Timeout	20

1.5.5 Sense Waiting Timeout	21

1.5.6 Enable/Disable Timeout between Decodes (Same Barcode)	22

1.5.7 Timeout Between Decodes (Same Barcode)	22

1.6 Command Triggering Mode	23

1.6.1 Mode Entry	23

1.6.2 Decode Session Timeout	24

2 Illumination & Aiming	24

2.1 Illumination	24

2.2 External Illumination	25

2.3 Aiming	26

3 Beep & LED Notifications	27

3.1 Buzzer	27

3.1.1 Passive Buzzer	27

3.1.2 Active Buzzer	27

3.2 All Beep	28

3.3 Startup Beep	28

3.4 Good Read Beep	29

3.4.1 Good Read Beep for Setting Barcode	29

3.4.2 Good Read Beep for Non-setting Barcode	30

3.4.3 Good Read Beep Timeout	30

3.5 Startup LED	31

3.5.1 Enable/Disable Startup LED	31

3.5.2 Startup LED Timeout	31

3.6 Good Read LED	32

3.6.1 Enable/Disable Good Read LED	32

3.6.2 Good Read LED Timeout	33

3.7 Good Read Vibration	33

3.7.1 Enable/Disable Good Read Vibration	33

3.7.2 Good Read Vibration Timeout	34

3.8 Read Fail Message	35

3.8.1 Transmit RF Message	35

3.8.2 Edit RF Message	35

4 Date Editing	36

4.1 Custom Prefix	36

4.2 AIM ID Prefix	38

4.3 Code ID Prefix	38

4.4 Data Segment Truncation	43

4.5 Custom Suffix	45

4.6 Tail Suffix	46

4.7 Transmit Protocol	47

4.8 HID KBW Output Compatibility	48

5 Symbologies	48

5.1 Enable/Disable All Symbologies	48

5.2 Video Reverse	49

5.3 EAN-13	50

5.3.1 Enable/Disable EAN-13	50

5.3.2 EAN-13 Transmit Check Digit	50

5.3.3 EAN-13 Add-On Code	51

5.3.4 EAN-13 Add-On Code Required	51

5.4 ISSN	52

5.4.1 Enable/Disable ISSN	52

5.4.2 ISSN Add-On Code	52

5.4.3 ISSN Add-On Code Required	53

5.5 ISBN	53

5.5.1 Enable/Disable ISBN	53

5.5.2 ISBN Add-On Code	53

5.5.3 ISBN Add-On Code Required	54

5.6 EAN-8	54

5.6.1 Enable/Disable EAN-8	54

5.6.2 EAN-8 Transmit Check Digit	55

5.6.3 EAN-8 Add-On Code	55

5.6.4 EAN-8 Add-On Code Required	56

5.6.5 EAN-8 to EAN-13	56

5.7 UPC-A	57

5.7.1 Enable/Disable UPC-A	57

5.7.2 UPC-A Transmit Check Digit	57

5.7.3 UPC-A Add-On Code	57

5.7.4 UPC-A Add-On Code Required	58

5.7.5 UPC-A to EAN-13	59

5.8 UPC-E	59

5.8.1 Enable/Disable UPC-E	59

5.8.2 UPC-E Transmit Check Digit	59

5.8.3 UPC-E Add-On Code	60

5.8.4 UPC-E Add-On Code Required	60

5.8.5 UPC-E to UPC-A	61

5.9 Code128	61

5.9.1 Enable/Disable Code128	61

5.9.2 Set Length for Code 128	62

5.10 Code39	62

5.10.1 Enable/Disable Code39	62

5.10.2 Set Length for Code39	63

5.10.3 Code39 Check Digit Verification (Modulo 43)	63

5.10.4 Code39 Transmit Start/Stop Character	64

5.10.5 Enable/Disable Code39 Full ASCII	65

5.11 Code32	65

5.11.1 Enable/Disable Code32	65

5.11.2 Code32 Transmit Check Digit	66

5.11.3 Code32 Transmit Start/Stop Character	66

5.11.4 Code32 Transmit Prefix Character	67

5.12 Code93	67

5.12.1 Enable/Disable Code93	67

5.12.2 Set Length for Code 93	67

5.12.3 Code93 Check Digit Verification	68

5.13 CodaBar	69

5.13.1 Enable/Disable CodaBar	69

5.13.2 Set Length for Codabar	69

5.13.3 CodaBar Check Digit Verification (Mod-16)	70

5.13.4 CodaBar Transmit Start/Stop Character	71

5.14 Interleaved 2 of 5	72

5.14.1 Enable/Disable Interleaved 2 of 5	72

5.14.2 Set Length for Interleaved 2 of 5	72

5.14.3 Interleaved 2 of 5 Check Digit Verification (Mod-10)	73

5.15 ITF-14	74

5.15.1 Enable/Disable ITF-14	74

5.16 ITF-6	74

5.16.1 Enable/Disable ITF-6	75

5.17 Industrial 2 of 5	75

5.17.1 Enable/Disable Industrial 2 of 5	75

5.17.2 Set Length for Industrial 2 of 5	75

5.17.3 Industrial 2 of 5 Check Digit Verification (Mod-10)	76

5.18 Matrix 2 of 5	77

5.18.1 Enable/Disable Matrix 2 of 5	77

5.18.2 Set Length for Matrix 2 of 5	77

5.18.3 Matrix 2 of 5 Check Digit Verification (Mod-10)	78

5.19 Code11	79

5.19.1 Enable/Disable Code11	79

5.19.2 Set Length for Code 11	79

5.19.3 Code11 Check Digit Verification	80

5.19.4 Code11 Transmit Check Digit	80

5.20 MSI-Plessey	81

5.20.1 Enable/Disable MSI-Plessey	81

5.20.2 Set Length for MSI-Plessey	81

5.20.3 MSI-Plessey Check Digit Verification	82

5.20.4 MSI-Plessey Transmit Check Digit	82

5.21 GS1-Databar (RSS)	83

5.21.1 Enable/Disable RSS-14	83

5.21.2 Enable/Disable RSS-Limited	83

5.21.3 Enable/Disable RSS-Expanded	84

5.21.4 Set Length for RSS	85

5.22 Micro QR Code	85

5.22.1 Enable/Disable Micro QR Code	85

5.23 QR Code	86

5.23.1 Enable/Disable QR Code	86

5.24 Data Matrix	86

5.24.1 Enable/Disable DM	86

5.25 PDF417	87

5.25.1 Enable/Disable PDF417	87

5.26 Micro PDF417	87

5.26.1 Enable/Disable Micro PDF417	87

5.27 Aztec Code	88

5.27.1 Enable/Disable Aztec Code	88

5.28 Maxi Code	88

5.28.1 Enable/Disable Maxi Code	88

6 Appendix C：Code ID Table	89

7 Appendix D：ASCII Table	90

8 Appendix E：Digit Barcodes	93

9 Appendix F：Save/Cancel Barcodes	95

Product introduction

Factory Default

Terminator：Enter(\r)。

Factory Default

Obtain device information

product version number

Wireless settings

Pairing setup code

- ①Pair the receiver: Scan the following two barcodes in sequence.And connect the receiver to computer；

2.4G MODE                                  Connecting devices

②Pair Bluetooth devices in HID mode: Scan the following two barcodes in sequence, turn on the Bluetooth device to search for RB_Scanner_HID connection.

HID MODE                                          Connect

BLE BLUETOOTH MODE

Pairing Bluetooth devices: Scan the following two barcodes in sequence, and perform Bluetooth search RB_Scanner_BLE on the software that supports BLE

Connect.Note: BLE mode needs to be output on specific software (Bluetooth serial port)

BLE MODE                                       Connect

Mode Selection

Instant upload mode                                Inventory Mode

Over-the-distance storage mode

①Operations in Inventory Mode

Upload all data                            Upload new data (that is, data that has not been uploaded before)

Display saved data                               Showing unuploaded data

Clear all data

Set the upload data speed delay

No delay                                        Delay 10ms

Delay 20ms

Check software version

Check the scanner software version             Check the receiving end version number

Set sleep time

X=1yyy(x=1000 means no sleep, sleep time calculation formula: yyy*10=z seconds)

No sleep                                         60 seconds

120 seconds                                       5 minutes

- 10 minutes

USB KBW

When the scanner is connected to the host using a USB receiver, the scanner can be configured as a standard keyboard input by scanning the USB KBW setup code.

USB KBW keyboard

USB COM keyboard

When the scanner is connected to the host using a USB receiver, the receiver virtual serial port output mode can be changed by scanning the USB COM setup code.

USB COM

Output format

UTF-8                                                      GBK

When the scanner recognizes it as a keyboard input device, different countries have different input characters, so you need to set different national languages. The keyboard defaults to the US language.

2.10 multi-language keyboards

U.S. (default)        Belgium (French)

Brazil (Portuguese)    Canada

Czech Republic         Denmark

Finland               France

Germany                  Italy

Portugal                 Spain

Türkiye-F                Türkiye-Q

U.K.                      Japan

Russia                   Vietnamese

Output forced letter case conversion

Letter conversion, when outputting barcodes with letter content, you can configure the output result to be all uppercase or all lowercase. For example, if the barcode content is: ab123dE, if you scan the "Convert to uppercase" barcode, the output result is: AB123DE; if you scan the "Convert to lowercase" barcode, the output result is: abc123de; the default case is not converted.

Do not convert                               Uppercase

Lowercase                          Case conversion

  - 三、Scanning module settings

  - Enable/Disable All Symbologies

Scanning the following setting barcodes will enable or disable symbologies. If disable all symbologies, only the setting barcodes can be read.

Enable All Symbologies

Disable All Symbologies

* Enable Default Symbologies

  - Video Reverse

In some special scenarios, the following barcodes can configure the engine to read regular and inverse barcodes. If enabled, it will affect the scanning speed. Please enable it in the desired scenario.

*1D Regular Symbologies

1D Regular and Inverse Symbologies

* Regular QR

Regular and Inverse QR

* Regular DM

Regular and Inverse DM

    - Trigger Mode

The manual mode can be configured to edge or level trigger by scanning the following barcodes.

*Level Trigger

Edge Trigger

    - Decode Session Timeout

In manual mode, this parameter refers to the maximum duration the decode session continues before good read. The range of decode session timeout is 0.1~25.5 seconds, with a step size of 0.1 seconds. When set to 0, it indicates an infinite reading time. The default duration is 5.0 seconds.

1000ms

3000ms

*5000ms

Infinite Duration

  - Continuous Mode

    - Mode Entry

After setting up, the engine immediately starts reading barcodes without trigger. When good read and outputs data or the decode session timeout expires, the engine will automatically start the next reading after waiting for a certain period of time (configurable). If the following situations do not occur, the engine will cycle operating in the above way: during the reading process, the user can also manually pause the reading by clicking the trigger button, and click the trigger button again to allow the engine to continue to cycle the reading.

Continuous mode

    - Decode Session Timeout

In continuous mode, this parameter refers to the maximum duration the decode session continues before good read. After good read or the decode session timeout expires, the scan engine will enter the interval period of not capturing and reading. The range of decode session timeout is 0.1~25.5 seconds, with a step size of 0.1 seconds; When set to 0, it indicates an infinite reading time. The default duration is 5.0 seconds.

1000ms

3000ms

*5000ms

Infinite Duration

    - Timeout Between Decodes

This parameter refers to the timeout between decode sessions, when a decode session ends, next session will not happen until the timeout between decodes expires. The setting range of timeout between decode is 0-25.5 seconds, with a step size of 0.1 seconds. The default interval duration is 1.0 seconds.

No Interval

500ms

*1000ms

1500ms

2000ms

    - Enable/Disable Timeout Between Decodes (Same Barcode)

Timeout between decodes (same barcode) can avoid undesired rereading of same barcode in a given period of time. The engine is only allowed to re-read same barcode after the timeout between decodes (same barcode) expires.

Enable Timeout Between Decodes

*Disable Timeout between Decodes

    - Timeout Between Decodes (Same Barcode)

When the timeout between decodes (same barcode) is enabled, the following setting barcodes can set the timeout between decodes for same barcode.

Infinite Delay

500ms

*1000ms

3000ms

5000ms

  - Sense Mode

    - Mode Entry

After setting up, the engine immediately starts monitoring the surrounding ambient illumination without trigger. When the scene changes, the engine starts reading until the image stabilization timeout expires. If the following situation does not occur, the engine will cycle operating in the above way: if the engine fails to scan the barcode within a decode session timeout, it will automatically pause reading and enter the monitoring mode. In sense mode, the scan engine can also start reading after the user presses the trigger button, and continue monitoring after good read and output data or when the user releases the trigger button.

Sense Mode

    - Sensitivity

Sensitivity refers to the degree of acuteness of the engine’s response to changes in ambient illumination in sense mode. When the engine judges that the degree of changes meets the requirements, it will switch from monitoring mode to scanning mode.

Low

Medium

*High

Enhanced

    - Image Stabilization Timeout

Image Stabilization Timeout refers to the time that the engine needs to wait for the image to stabilize between detecting scene changes and starting to read barcodes in sense mode. The range of image stabilization timeout is 0-25.5 seconds, with a step size of 0.1 seconds. The default image stabilization timeout is 0 seconds.

*0ms

100ms

400ms

1000ms

2000ms

    - Decode Session Timeout

In sense mode, this parameter refers to the maximum duration the decode session continues before good read. After good read or the decode session timeout expires, the scan engine will enter the interval period of not capturing and reading. The range of decode session timeout is 0.1~25.5 seconds, with a step size of 0.1 seconds. When set to 0, it indicates an infinite reading time. The default duration is 5.0 seconds.

1000ms

3000ms

*5000ms

Infinite Duration

    - Sense Waiting Timeout

Sense waiting timeout refers to the time that the engine needs to wait for re sense after good read or the decode session timeout expires in sense mode. The setting range for sense waiting timeout is 0-25.5 seconds, with a step size of 0.1 seconds. The default duration is 0 seconds.

*0ms

500ms

1000ms

2000ms

5000ms

    - Enable/Disable Timeout between Decodes (Same Barcode)

Timeout between Decodes (Same Barcode) can avoid undesired rereading of same barcode in a given period of time. The engine is only allowed to re-read same barcode after the timeout between decodes (same barcode) expires.

Enable Timeout between Decodes

*Disable Timeout between Decodes

    - Timeout Between Decodes (Same Barcode)

When the timeout between decodes (same barcode) is enabled, the following setting barcodes can set the timeout between decodes for same barcode.

Infinite delay

500ms

*1000ms

3000ms

5000ms

  - Command Triggering Mode

    - Mode Entry

In this mode, the engine starts reading barcodes when it receives a scan command sent by the host (Write "1" to bit0 of flag bit 0x0002), and stops reading after good read and output data or the decode session timeout expires.

Command triggering mode

Note：In command triggering mode, the serial port command that triggers scanning is: 7E 00 08 01 00 02 01 AB CD; After receiving the trigger command, the scan engine will first output a seven byte response message and synchronously start scanning (response message content: 02 00 00 01 00 33 31).

    - Decode Session Timeout

In command triggered mode, this parameter refers to the maximum duration the decode session continues before good read. The range of decode session timeout is 0.1~25.5 seconds, with a step size of 0.1 seconds. When set to 0, it indicates an infinite reading time. The default duration is 5.0 seconds.

1000ms

3000ms

*5000ms

Infinite Duration

- Illumination & Aiming

  - Illumination

Illumination can provide auxiliary lighting during every image capture, with a beam of light shining on the target barcode, improving reading performance and adaptability to weak ambient Illumination. Users can set it to one of the following states according to the application environment：

Illumination Normal（default）：The Illumination LEDs turn on during image capture, and turn off at other times;

Illumination Always On：The Illumination LEDs keep on after the engine is powered on;

Illumination Off：The Illumination LEDs keep off all the time.

*Illumination Normal

Illumination Always On

Illumination Off

  - External Illumination

Users can control the external illumination by scanning the following setting barcodes (hardware connection method can be found in the hardware reference manual).

*Disable External Illumination

Enabled External Illumination

*External Illumination High-level Effective

External Illumination Low-level Effective

  - Aiming

The projected aiming pattern can help users find the optimal reading position during image capture. The user can select any of the following modes according to the application environment:

Aiming Normal:（default）：The engine projects an aiming pattern only during image capture;

Aiming Always ON：Aiming beam is constantly on after the engine is powered on;

Aiming Off：Aiming pattern is off all the time.

*Aiming Normal

Aiming Always ON

Aiming Off

- Beep & LED Notifications

  - Buzzer

    - Passive Buzzer

Scan the following setting barcodes to set the driving frequency of the passive buzzer and achieve volume control.

Passive Buzzer - Low Tone

*Passive Buzzer - Medium Tone

Passive Buzzer - High Tone

    - Active Buzzer

Active buzzer

In active buzzer mode, users can scan the "Buzzer Working Level-High" barcode to set the buzzer to be: idle low level and working high level, and scan the "Buzzer Working Level-Low" barcode to set the buzzer to be: idle high level and working low level.

*Buzzer Working Level - High

Buzzer Working Level - Low

  - All Beep

Scan the "Enable Mute" barcode to turn off all beep notifications and scan the "Disable Mute" barcode to cancel mute settings.

Enable Mute

*Disable Mute

  - Startup Beep

Scanning the "Enable Startup Beep" barcode can enable the startup beep. Scan the "Disable Startup Beep" barcode to disable the startup beep.

*Enable Startup Beep

Disable Startup Beep

  - Good Read Beep

The good read beep includes good read beep for setting barcode and good read beep for Non- setting barcode.

*Enable Good Read Beep

Disable Good Read Beep

    - Good Read Beep for Setting Barcode

In the [Enable Good Read Beep] state, the following setting barcodes can be scanned separately to enable or disable the good read beep for setting barcode.

*Enable Good Read Beep for Setting Barcode

Disable Good Read Beep for Setting Barcode

    - Good Read Beep for Non-setting Barcode

In the [Enable Good Read Beep] state, the following setting barcodes can be scanned separately to enable or disable the good read beep for Non-setting barcode.

*Enable Good Read Beep for Non-setting Barcode

Disable Good Read Beep for Non-setting Barcode

    - Good Read Beep Timeout

Scanning the "Beep Timeout" barcodes can set good read beep timeout, with a default of 60ms.

Good Read Beep Timeout-30ms

* Good Read Beep Timeout-60ms

Good Read Beep Timeout-90ms

Good Read Beep Timeout-120ms

  - Startup LED

    - Enable/Disable Startup LED

Scanning the "Enable Startup LED" barcode can enable the engine to output a high-level pulse on the DLED pin of the 12PIN connector after startup. Scan the "Disable Startup LED" barcode to cancel the high-level pulse output of the DLED pin.

Enable Startup LED

*Disable Startup LED

    - Startup LED Timeout

Startup LED Timeout-100ms

*Startup LED Timeout-200ms

Startup LED Timeout-300ms

Startup LED Timeout-500ms

Startup LED Timeout-1000ms

Startup LED Timeout-2000ms

  - Good Read LED

    - Enable/Disable Good Read LED

Scanning the "Enable Good Read LED" barcode can enable the engine to output a high-level pulse on the DLED pin of the 12PIN connector after good read. Scanning the "Disable Good Read LED" barcode will cancel the high-level pulse output on the DLED pin.

*Enable Good Read LED

Disable Good Read LED

    - Good Read LED Timeout

Good Read LED Timeout-30ms

* Good Read LED Timeout-60ms

Good Read LED Timeout-90ms

Good Read LED Timeout-120ms

Good Read LED Timeout-200ms

Good Read LED Timeout-500ms

  - Good Read Vibration

    - Enable/Disable Good Read Vibration

Enable Good Read Vibration

*Disable Good Read Vibration

Vibration High-level Effective

*Vibration Low-level Effective

    - Good Read Vibration Timeout

* Good Read Vibration Timeout-100ms

Good Read Vibration Timeout-200ms

Good Read Vibration Timeout-300ms

Good Read Vibration Timeout-400ms

  - Read Fail Message

    - Transmit RF Message

RF (Read fail) message settings can enable the engine whether or not to transmit a user-defined RF message when a barcode is not good read. Users or programs can make corresponding adjustments or operations after detecting this message.

Transmit RF Message

*Do Not Transmit RF Message

    - Edit RF Message

Users can scan the "Edit RF Message" barcode and the digit barcodes to edit RF message. Each RF character is represented by two hexadecimal values, with a maximum of 15 characters allowed in RF. The hexadecimal conversion table for character values can be referenced (Appendix D: ASCII Table).

Edit RF Message

Example：Edit user-defined RF Message to "FAIL"

- According to the character table (Appendix D: ASCII Table), the hexadecimal values for the four characters in "FAIL" are: "46", "41", "49", "4C";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "Edit RF Message" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "4", "6", "4", "1", "4", "9", "4", "C";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes).

- Date Editing

In practical applications, sometimes the barcode data needs to be edited and distinguished from one another.

Data editing includes：

- Append custom prefix;

- Append AIM ID;

- Append Code ID;

- Append custom suffix;

- Decoded Data segment truncation;

- Append Tail suffix.

The default output order of edited data is as follows：

【Prefix】【AIM ID】【Code ID】【Data】【Suffix】【Tail】

  - Custom Prefix

Enable/ Disable Custom Prefix

Prefix is the string that can be customized and modified by the user before the decoded data and can be enabled by scanning the "Enable Custom Prefix" barcode.

Enable Custom Prefix

*Disable Custom Prefix

Modify Custom Prefix

Users can scan the "Modify Custom Prefix" barcode and the digit barcodes to modify the prefix. Each prefix character is represented by two hexadecimal values, with a maximum of 15 characters allowed in prefix. The hexadecimal conversion table for character values can be referenced (Appendix D: ASCII Table).

Modify Custom Prefix

Example：Modify the custom prefix to "DATA"

- According to the character table (Appendix D: ASCII Table), the hexadecimal values of the four characters in "DATA" are: "44", "41", "54", "41";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "Modify Custom Prefix" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "4", "4", "4", "1", "5", "4", "4", "1";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes).

  - AIM ID Prefix

AIM is the abbreviation for Automatic Identification Manufacturers. The AIM IDs define symbology identifiers (Users cannot customize the AIM ID). The specific definition can be found in Appendix B: AIM ID Table. If AIM ID Prefix is enabled, the engine will add the symbology identifier before the barcode data after decoding with the following format: "]"+letter "C"+ number "0". For example, the AIM ID of Code 128 is "]C0".

Enable/ Disable AIM ID Prefix

Users can identify different barcode types through AIM IDs, but the corresponding AIM ID for each barcode type cannot be modified by users. The AIM ID consists of three characters.

Enable AIM ID Prefix

*Disable AIM ID Prefix

  - Code ID Prefix

Enable/ Disable Code ID Prefix

Users can also identify different barcode types through Code IDs, and the corresponding Code ID for each barcode type can be modified by the users. The Code ID consists of one character.

Enable Code ID Prefix

*Disable Code ID Prefix

Restore All Default Code IDs

Scan the "Restore All Default Code IDs" barcode, the corresponding Code ID for each barcode can be restored to the default value. The default Code ID can be referred to (Appendix C: Code ID Table).

*Restore All Default Code IDs

Modify Code ID

Users can modify the Code ID corresponding to each barcode by scanning the corresponding barcode and the digit barcodes. The Code ID is represented by one hexadecimal value, and the hexadecimal conversion table for character values can be referred to (Appendix D: ASCII Table).

Example：Modify the Code128 Code ID to "A"

- According to the character table (Appendix D: ASCII Table), the hexadecimal value of character "A" is "41";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" code (see Chapter 2.1);

- Scan the "Modify Code128 Code ID " barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "4", "1";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes).

Modifying Code ID Setting Barcodes List:

Modify Aztec Code ID

Modify Codabar Code ID

Modify Code39 Code ID

Modify Code93 Code ID

Modify Code128 Code ID

Modify GS1 DataBar(RSS14) Code ID

Modify GS1 DataBar Expanded Code ID

Modify Data Matrix Code ID

Modify EAN-8 Code ID

Modify EAN-13 Code ID

Modify Interleaved 2 of 5 Code ID

Modify Maxi Code ID

Modify PDF417 Code ID

Modify QR Code ID

Modify UPC-A Code ID

Modify UPC-E Code ID

Modify Micro QR Code ID

Modify MSI Plessey Code ID

Modify Industrial 2 of 5 Code ID

Modify Matrix 2 of 5 Code ID

Modify Code11 Code ID

Modify ISSN Code ID

Modify ISBN Code ID

Modify GS1 DataBar Limited Code ID

Modify Micro PDF417 Code ID

Modify Code32 Code ID

  - Data Segment Truncation

This function can be enabled when the user only needs to output a portion of the decoding data.

The decode information [Data] can be divided into three parts: [Start], [Center], and [End].

The character length of the Start and End segments can be configured by scanning barcodes.

Users can choose to output decoding information at the corresponding position by scanning the following setting barcodes.

* Transmit Entire Data Segment

Only Transmit Start Segment

Only Transmit End Segment

Only Transmit Center Segment

Modify Start Truncation Length M

Users can scan the "Start Truncation Length M" barcode and the digit barcodes to modify the length of the Start segment. The truncation length M is represented by one hexadecimal character, with a maximum of 255 characters allowed in Start segment. The hexadecimal value conversion table corresponding to the length M can be referred to (Appendix D: ASCII Table).

Start Truncation Length M

Modify End Truncation Length N

Users can scan the "End Truncation Length N" barcode and the digit barcodes to modify the length of the End segment. The truncation length N is represented by one hexadecimal character, with a maximum of 255 characters allowed in End segment. The hexadecimal value conversion table corresponding to the length N can be referred to (Appendix D: ASCII Table).

End Truncation Length N

Only Transmit Start Segment

Example：When decoding information is "1234567890123ABC", output the first thirteen bytes "1234567890123"

- According to the character table (Appendix D: ASCII Table), the hexadecimal character corresponding to decimal data "13" is "0D";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "Start Truncation Length M" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "0", "D";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes);

- Scan the "Only Transmit Start Segment" barcode.

Only transmit End segment

Example：When decoding information is "1234567890123ABC", output the last three bytes "ABC"

- According to the character table (Appendix D: ASCII Table), the hexadecimal character corresponding to decimal data "3" is "03";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "End Truncation Length N" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "0", "3";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes);

- Scan the "Only Transmit End Segment" barcode.

Only transmit the Center segment

Example：When decoding information is "12345678900123ABC", output the middle four bytes "0123"

- According to the character table (Appendix D: ASCII Table), the hexadecimal characters corresponding to decimal data "10" and "3" are "0A" and "03";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "End Truncation Length N" setting barcode；

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "0", "3";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes);

- Scan the "Start Truncation Length M" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "0", "A";

- Scan the "Save" barcode (Appendix F: Save/ Cancel Barcodes);

- Scan the "Only Transmit Center Segment" barcode.

  - Custom Suffix

Enable/ Disable Custom Suffix

The suffix is the string that can be customized and modified by the user after the decoded data and can be enabled by scanning the "Enable Custom Suffix" barcode.

Enable Custom Suffix

*Disable Custom Suffix

Modify Custom Suffix

Users can scan the "Modify Custom Suffix" barcode and the digit barcodes to modify the suffix. Each suffix character is represented by two hexadecimal values, with a maximum of 15 characters allowed in suffix. The hexadecimal conversion table for character values can be referred to (Appendix D: ASCII Table).

Modify Custom Suffix

Example：Modify the Custom Suffix to "DATA"

- According to the character table (Appendix D: ASCII Table), the hexadecimal values of the four characters in "DATA" are: "44", "41", "54", "41";

- Confirm whether the setting barcode is enabled. If it is not enabled, please scan the "Enter Setup" barcode (see Chapter 2.1);

- Scan the "Modify Custom Suffix" barcode;

- Scan the digit barcodes in sequence (Appendix E: Digit Barcodes) "4", "4", "4", "1", "5", "4", "4", "1";

- Scan the "Save" barcode (Appendix F: Save/Cancel Barcodes).

  - Tail Suffix

To enable the host to quickly distinguish the current decoding result, this function can be enabled.

After scanning the "Modify Tail Suffix" barcodes, if good read, the engine will add the corresponding tail suffix after the decoded data.

Disable Tail Suffix

*Modify Tail Suffix to CR

Modify Tail Suffix to TAB

Modify Tail Suffix to CRLF

Modify Tail Suffix to Two CRLF

Modify Tail Suffix to CRDownArrow

  - Transmit Protocol

By scanning the following settings barcode, the transmit format of decoded data in serial/virtual serial mode can be modified.

Among them, the transmit format with protocol is: <03><length><decoded data>.

*Raw Data

With Protocol

  - HID KBW Output Compatibility

Users can switch the compatibility of HID KBW by scanning the following barcodes.

*Compatibility Mode 1

Compatibility Mode 2

- Symbologies

  - Enable/Disable All Symbologies

Scanning the following barcodes will enable or disable symbologies. If all symbologies disabled, only the setting barcodes can be read.

Enable All Symbologies

Disable All Symbologies

* Enable Default Symbologies

  - Video Reverse

In some special scenarios, the following barcodes can configure the engine to read regular and inverse barcodes. If enabled, it will affect the reading speed. Please enable it in the desired scenario.

*1D Regular Symbologies

1D Regular and Inverse Symbologies

* Regular QR

Regular and Inverse QR

* Regular DM

Regular and Inverse DM

  - EAN-13

    - Enable/Disable EAN-13

Scan the following barcodes to enable/disable EAN-13.

*Enable EAN-13

Disable EAN-13

    - EAN-13 Transmit Check Digit

EAN-13 is 13 digits in length with the last one as its check digit used to verify the integrity of the data. The EAN-13 transmission check digit function can be enabled or disabled by scanning the following setting barcodes.

*Transmit EAN-13 Check Digit

Do Not Transmit EAN-13 Check Digit

    - EAN-13 Add-On Code

Scan the following setting barcodes to enable or disable the EAN-13 add-on code.

*Disable 2-Digit Add-On Code

Enable 2-Digit Add-On Code

*Disable 5-Digit Add-On Code

Enable 5-Digit Add-On Code

    - EAN-13 Add-On Code Required

Scan the following setting barcodes to configure the engine to read EAN-13 barcodes that whether or not need to contain add-on code.

*EAN-13 Add-On Code Not Required

EAN-13 Add-On Code Required

  - ISSN

    - Enable/Disable ISSN

Scan the following setting barcodes to enable /disable ISSN.

Enable ISSN

*Disable ISSN

    - ISSN Add-On Code

Scan the following setting barcodes to enable or disable the ISSN add-on code.

*Disable 2-Digit Add-On Code

Enable 2-Digit Add-On Code

    - ISSN Add-On Code Required

Scan the following setting barcodes to configure the engine to read ISSN barcodes that whether or not need to contain add-on code.

*ISSN Add-On Code Not Required

ISSN Add-On Code Required

  - ISBN

    - Enable/Disable ISBN

Scan the following setting barcodes to enable/disable ISBN.

Enable ISBN

*Disable ISBN

    - ISBN Add-On Code

Scan the following setting barcodes to enable or disable the ISBN add-on code.

*Disable 5-Digit Add-On Code

Enable 5-Digit Add-On Code

    - ISBN Add-On Code Required

Scan the following setting barcodes to configure the engine to read ISBN barcodes that whether or not need to contain add-on code.

*ISBN Add-On Code Not Required

ISBN Add-On Code Required

  - EAN-8

    - Enable/Disable EAN-8

Scan the following setting barcodes to enable/disable EAN-8.

*Enable EAN-8

Disable EAN-8

    - EAN-8 Transmit Check Digit

EAN-8 is 8 digits in length with the last one as its check digit used to verify the integrity of the data. The EAN-8 transmission check digit function can be enabled or disabled by scanning the following setting barcodes.

*Transmit EAN-8 Check Digit

Do Not Transmit EAN-8 Check Digit

    - EAN-8 Add-On Code

Scan the following setting barcodes to enable or disable the EAN-8 add-on code.

*Disable 2-Digit Add-On Code

Enable 2-Digit Add-On Code

*Disable 5-Digit Add-On Code

Enable 5-Digit Add-On Code

    - EAN-8 Add-On Code Required

Scan the following setting barcodes to configure the engine to read EAN-8 barcodes that whether or not need to contain add-on code.

* EAN-8 Add-On Code Not Required

EAN-8 Add-On Code Required

    - EAN-8 to EAN-13

The function of converting EAN-8 to EAN-13 can be enabled or disabled by scanning the following setting barcodes.

Enable EAN-8 to EAN-13

*Disable EAN-8 to EAN-13

  - UPC-A

    - Enable/Disable UPC-A

Scan the following setting barcodes to enable/disable UPC-A.

*Enable UPC-A

Disable UPC-A

    - UPC-A Transmit Check Digit

UPC-A is 13 digits in length with the last one as its check digit used to verify the integrity of the data. The UPC-E transmission check digit function can be enabled or disabled by scanning the following setting barcodes.

*Transmit UPC-A Check Digit

Do Not Transmit UPC-A Check Digit

    - UPC-A Add-On Code

Scan the following setting barcodes to enable or disable the UPC-A add-on code.

*Disable 2-Digit Add-On Code

Enable 2-Digit Add-On Code

*Disable 5-Digit Add-On Code

Enable 5-Digit Add-On Code

    - UPC-A Add-On Code Required

Scan the following setting barcodes to configure the transmit mode of UPC-A.

* UPC-A without Add-On Code

UPC-A with Add-On Code

UPC-A Add-On Code Not Required

    - UPC-A to EAN-13

The function of converting UPC-A to EAN-13 can be enabled or disabled by scanning the following setting barcodes.

Enable UPC-A to EAN-13

*Disable UPC-A to EAN-13

  - UPC-E

    - Enable/Disable UPC-E

Scan the following setting barcodes to enable/disable UPC-E.

*Enable UPC-E

Disable UPC-E

    - UPC-E Transmit Check Digit

UPC-E is 8 digits in length with the last one as its check digit used to verify the integrity of the data. The UPC-E transmission check digit function can be enabled or disabled by scanning the following setting barcodes.

*Transmit UPC-E Check Digit

Do Not Transmit UPC-E Check Digit

    - UPC-E Add-On Code

Scan the following setting barcodes to enable or disable the UPC-E add-on code.

*Disable 2-Digit Add-On Code

Enable 2-Digit Add-On Code

*Disable 5-Digit Add-On Code

Enable 5-Digit Add-On Code

    - UPC-E Add-On Code Required

Scan the following setting barcodes to configure the engine to read UPC-E barcodes that whether or not need to contain add-on code.

* UPC-E Add-On Code Not Required

UPC-E Add-On Code Required

    - UPC-E to UPC-A

The function of converting UPC-E to UPC-A can be enabled or disabled by scanning the following setting barcodes.

Enable UPC-E to UPC-A

*Disable UPC-E to UPC-A

  - Code128

    - Enable/Disable Code128

Scan the following setting barcodes to enable/disable Code128.

*Enable Code128

Disable Code128

    - Set Length for Code 128

Scan the following setting barcodes to set the minimum length for Code128.

Set the Minimum Length to 0

*Set the Minimum Length to 4

  - Code39

    - Enable/Disable Code39

Scan the following setting barcodes to enable/disable Code39.

* Enable Code39

Disable Code39

    - Set Length for Code39

Scan the following setting barcodes to set the minimum length for Code39.

Set the Minimum Length to 0

*Set the Minimum Length to 4

    - Code39 Check Digit Verification (Modulo 43)

Code 39 does not require the inclusion of check digit in barcode data. If there is, it must be the last byte of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Code39 Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the normal data except for the last check digit will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification ", the scanner will verify according to the last digit of the barcode data. If the verification passes, the check digit will be transmitted together as the last digit of normal data. If the verification fails, the barcode content will not be sent.

*Disable Code39 Verification

Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 1 byte of check digit is less than the minimum length limit, the barcode reading will fail.

For example, in the current scanner settings, the minimum length for Code 39 is 4 bytes, if do not transmit check digit, reading Code 39 with a total length of 4 bytes will fail！

    - Code39 Transmit Start/Stop Character

Scan the following setting barcodes to configure the start and stop character output of Code39.

*Do Not Transmit Start Character

Transmit Start Character

*Do Not Transmit Stop Character

Transmit Stop Character

    - Enable/Disable Code39 Full ASCII

Scan the “Enable Code39 Full ASCII” barcode to configure the engine to read all ASCII characters.

*Disable Code39 Full ASCII

Enable Code39 Full ASCII

  - Code32

    - Enable/Disable Code32

Scan the following setting barcodes to enable/disable Code32.

Enable Code32

*Disable Code32

    - Code32 Transmit Check Digit

Do Not Transmit Code32 Check Digit

*Transmit Code32 Check Digit

    - Code32 Transmit Start/Stop Character

Scan the following setting barcodes to configure the start/stop character output of Code32.

*Do Not Transmit Start Character

Transmit Start Character

*Do Not Transmit Stop Character

Transmit Stop Character

    - Code32 Transmit Prefix Character

Scan the following setting barcodes to configure the prefix character output of Code32.

Transmit Prefix Character A

*Do Not Transmit Prefix Character A

  - Code93

    - Enable/Disable Code93

Scan the following setting barcodes to enable/disable Code93.

*Enable Code 93

Disable Code 93

    - Set Length for Code 93

Scan the following setting barcodes to set the minimum length for Code93.

Set the Minimum Length to 0

*Set the Minimum Length to 4

    - Code93 Check Digit Verification

Code93 barcode data does not require the inclusion of check digit. If there is, they must be the last two digits of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Code93 Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last two digits of the barcode data. If the verification passes, the normal data except for the last two check digits will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification", the scanner will verify according to the last two digits of the barcode data. If the verification passes, the check digits will be transmitted together as the last two digits of normal data. If the verification fails, the barcode content will not be sent.

Disable Code93 Verification

*Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 2 bytes of check digits is less than the minimum length limit, the barcode reading will fail.

For example, in the current scanner settings, the minimum length for Code93 is 4 bytes, if do not transmit check digit, reading Code93 with a total length of 4 bytes will fail！

  - CodaBar

    - Enable/Disable CodaBar

Scan the following setting barcodes to enable/disable CodaBar.

* Enable Codabar

Disable Codabar

    - Set Length for Codabar

Scan the following setting barcodes to set the minimum length for CodaBar.

Set the Minimum Length to 2

*Set the Minimum Length to 4

    - CodaBar Check Digit Verification (Mod-16)

Codebar barcode data does not require the inclusion of check digit. If there is, it must be the last byte of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Codabar Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the normal data except for the last check digit will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the check digit will be transmitted together as the last digit of normal data. If the verification fails, the barcode content will not be sent.

*Disable Codabar Verification

Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 1 byte of check digit is less than the minimum length limit, the barcode reading will fail.

For example, in the current scanner settings, if the minimum length for Codabar is 4 bytes, if do not transmit check digit, reading a Codabar with a total length of 4 bytes will fail！

    - CodaBar Transmit Start/Stop Character

There is one byte data before and after the Codabar barcode data as the start character and the stop character. The start character and the stop character are one of the four characters "A", "B", "C", and "D". Users can set whether to transmit the start character and stop character together with the barcode data after good read.

Scan the following setting barcodes to set the start and stop characters sending of CodaBar.

Transmit Start/Stop Character

*Do Not Transmit Start/Stop Character

  - Interleaved 2 of 5

    - Enable/Disable Interleaved 2 of 5

Scan the following setting barcodes to enable/disable Interleaved 2 of 5.

Enable Interleaved 2 of 5

*Disable Interleaved 2 of 5

    - Set Length for Interleaved 2 of 5

Scan the following setting barcodes to set the minimum length for Interleaved 2 of 5.

Set the Minimum Length to 0

*Set the Minimum Length to 6

    - Interleaved 2 of 5 Check Digit Verification (Mod-10)

Interleaved 2 of 5 barcode data does not require the inclusion of check digit. If there is, it must be the last byte of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Interleaved 2 of 5 Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the normal data except for the last check digit will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the check digit will be transmitted together as the last digit of normal data. If the verification fails, the barcode content will not be sent.

The length of the Interleaved 2 of 5 barcode must be even, incudes check digit.. If the length is odd, a “0” is added before the first digit. The check digit is automatically generated during coding.

*Disable Interleaved 2 of 5 Verification

Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 1 byte of check digit is less than the minimum length limit, the barcode reading will fail.

For example, in the current scanner settings, the minimum length for Interleaved 2 of 5 is 4 bytes. If do not transmit check digit, reading an Interleaved 2 of 5 with a total length of 4 bytes will fail！

  - ITF-14

ITF-14 is a specific format of Interleaved 2 of 5, it is 14 digits in length with the last one as its check digit used to verify the integrity of the data.

    - Enable/Disable ITF-14

Scan the following setting barcodes to enable/disable ITF-14.

Enable ITF-14

*Disable ITF-14

  - ITF-6

ITF-6 is similar to ITF-14 as a specific format of Interleaved 2 of 5. It is 6 digits in length with a fixed requirement for verification.

    - Enable/Disable ITF-6

Scan the following setting barcodes to enable/disable ITF-6.

Enable ITF-6

*Disable ITF-6

  - Industrial 2 of 5

    - Enable/Disable Industrial 2 of 5

Scan the following setting barcodes to enable/disable Industrial 2 of 5.

Enable Industrial 2 of 5

*Disable Industrial 2 of 5

    - Set Length for Industrial 2 of 5

Scan the following setting barcodes to set the minimum length for Industrial 2 of 5.

Set the Minimum Length to 0

*Set the Minimum Length to 6

    - Industrial 2 of 5 Check Digit Verification (Mod-10)

The Industrial 25 barcode data does not require the inclusion of check digit. If there is, it must be the last byte of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Industrial 2 of 5 Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the normal data except for the last check digit will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the check digit will be transmitted together as the last digit of normal data. If the verification fails, the barcode content will not be sent.

*Disable Industrial 2 of 5 Verification

Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 1 byte of checksum characters is less than the minimum length limit, the barcode reading will fail.

For example, if the minimum length for Industrial 25 in the current scanner settings is 4 bytes and do not transmit check digit, reading Industrial 25 with a total length of 4 bytes will fail！

  - Matrix 2 of 5

    - Enable/Disable Matrix 2 of 5

Scan the following setting barcodes to enable/disable Matrix 2 of 5.

Enable Matrix 2 of 5

*Disable Matrix 2 of 5

    - Set Length for Matrix 2 of 5

Scan the following setting barcodes to set the minimum length for Matrix 2 of 5.

Set the Minimum Length to 0

*Set the Minimum Length to 6

    - Matrix 2 of 5 Check Digit Verification (Mod-10)

Matrix 2 of 5 barcode data does not require the inclusion of check digit. If there is, it must be the last byte of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

- If set to "Disable Matrix 25 Verification", the scanner will transmit all barcode data normally.

- Set to "Do Not Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the normal data except for the last check digit will be transmitted. If the verification fails, the barcode content will not be sent.

- If set to "Transmit Check Digit after Verification", the scanner will verify according to the last digit of the barcode data. If the verification passes, the check digit will be transmitted together as the last digit of normal data. If the verification fails, the barcode content will not be sent.

*Disable Matrix 25 Verification

Do Not Transmit Check Digit after Verification

Transmit Check Digit after Verification

When set to "Do Not Transmit Check Digit after Verification", if the data length that minus 1 byte of check digit is less than the minimum length limit, the barcode reading will fail.

For example, if the minimum length for Matrix 2 of 5 in the current scanner settings is 4 bytes and do not transmit check digit, reading Matrix 2 of 5 with a total length of 4 bytes will fail！

  - Code11

    - Enable/Disable Code11

Scan the following setting barcodes to enable/disable Code11.

Enable Code 11

*Disable Code 11

    - Set Length for Code 11

Scan the following setting barcodes to set the minimum length for Code11.

Set the Minimum Length to 0

*Set the Minimum Length to 4

    - Code11 Check Digit Verification

Code 11 barcode data does not require the inclusion of check digit. If there is, it can be the last 1 or 2 bytes of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

Therefore, if select "Disable Code11 Verification", the scanner will transmit all barcode data normally.

*Disable Code11 Verification

Code11 One Check Digit

Code11 Two Check Digits

    - Code11 Transmit Check Digit

Do Not Transmit Check Digit

*Transmit Check Digit

  - MSI-Plessey

    - Enable/Disable MSI-Plessey

Scan the following setting barcodes to enable/disable MSI Plessey.

Enable MSI-Plessey

*Disable MSI-Plessey

    - Set Length for MSI-Plessey

Scan the following setting barcodes to set the minimum length for MSI.

*Set the Minimum Length to 0

Set the Minimum Length to 4

    - MSI-Plessey Check Digit Verification

MSI Plessey barcode data does not require the inclusion of check digit. If there is, it is the last 1 or 2 characters of the data. The check digit value is calculated from all data except for the check digit, used to verify the integrity of the data.

If select "Disable MSI Verification", the scanner will transmit all barcode data normally.

Disable MSI Verification

*MSI One Check Digit

MSI Two Check Digits

    - MSI-Plessey Transmit Check Digit

*Transmit Check Digit

Do Not Transmit Check Digit

When verification is enabled and set to "Do Not Transmit Check Digit", if the data length that minus 2 bytes of check digit is less than the minimum length limit, the reading will fail.

For example, in the current scanner settings, the minimum length for MSI Plessey is 4 bytes, and if do not transmit check digit, reading MSI Plessey with a total length of 4 bytes will fail！

  - GS1-Databar (RSS)

    - Enable/Disable RSS-14

Scan the following setting barcodes to enable/disable RSS-14.

Enable RSS-14

*Disable RSS-14

*Transmit Application Identifier “01”

Do Not Transmit Application Identifier “01”

    - Enable/Disable RSS-Limited

Scan the following setting barcodes to enable/disable RSS-Limited.

Enable RSS-Limited

*Disable RSS-Limited

*Transmit Application Identifier“01”

Do Not Transmit Application Identifier “01”

    - Enable/Disable RSS-Expanded

Scan the following setting barcodes to enable/disable RSS-Expanded.

Enable RSS-Expanded

*Disable RSS-Expanded

*Transmit Application Identifier “01”

Do Not Transmit Application Identifier “01”

    - Set Length for RSS

Scan the following setting barcodes to set the minimum length for RSS.

Set the Minimum Length to 0

*Set the Minimum Length to 4

  - Micro QR Code

    - Enable/Disable Micro QR Code

Scan the following setting barcodes to enable or disable Micro QR code.

Enable Micro QR Code

*Disable Micro QR Code

  - QR Code

    - Enable/Disable QR Code

Scan the following setting barcodes to enable/disable QR code.

*Enable QR Code

Disable QR Code

  - Data Matrix

    - Enable/Disable DM

Scan the following setting barcodes to enable/disable DM.

*Enable DM

Disable DM

  - PDF417

    - Enable/Disable PDF417

Scan the following setting barcodes to enable/disable PDF417.

*Enable PDF417

Disable PDF417

  - Micro PDF417

    - Enable/Disable Micro PDF417

Scan the following setting barcodes to enable/disable Micro PDF417.

Enable Micro PDF417

*Disable Micro PDF417

  - Aztec Code

    - Enable/Disable Aztec Code

Scan the following setting barcodes to enable/disable Aztec Code.

Enable Aztec

*Disable Aztec

  - Maxi Code

    - Enable/Disable Maxi Code

Scan the following setting barcodes to enable/disable Maxi Code.

Enable Maxi

*Disable Maxi

- Appendix C：Code ID Table

Table 11-1  Code ID Table

| Symbology | Code ID |
| --- | --- |
| Aztec | z |
| Codabar | a |
| Code39 | b |
| Code93 | i |
| Code128 | j |
| GS1 DataBar(RSS14) | R |
| GS1 DataBar Expanded | R |
| Data Matrix | u |
| EAN-8 | d |
| EAN-13 | d |
| Interleaved 2 of 5 | e |
| Maxi | x |
| PDF417 | r |
| QR | Q |
| UPC-A | c |
| UPC-E | c |
| Micro QR | Q |
| MSI Plessey | m |
| Industrial 2 of 5 | I |
| Matrix 2 of 5 | v |
| Code 11 | H |
| ISSN | g |
| ISBN | B |
| GS1 DataBar Limited | R |
| Micro PDF417 | s |
| Code32 | b |

- Appendix D：ASCII Table

Table 12-1  ASCII Table

| Hex | Dec | Char |
| --- | --- | --- |
| 00 | 0 | NUL (Null Char.) |
| 01 | 1 | SOH (Start Of Header) |
| 02 | 2 | STX (Start Of Text) |
| 03 | 3 | ETX (End Of Text) |
| 04 | 4 | EOT (End Of Transmission) |
| 05 | 5 | ENQ (Enquiry) |
| 06 | 6 | ACK (Acknowledgment) |
| 07 | 7 | BEL (Bell) |
| 08 | 8 | BS (Backspace) |
| 09 | 9 | HT (Horizontal Tab) |
| 0a | 10 | LF (Line Feed) |
| 0b | 11 | VT (Vertical Tab) |
| 0c | 12 | FF (Form Feed) |
| 0d | 13 | CR (Carriage Return) |
| 0e | 14 | SO (Shift Out) |
| 0f | 15 | SI (Shift In) |
| 10 | 16 | DLE (Data Link Escape) |
| 11 | 17 | DC1 (XON) (Device Control 1) |
| 12 | 18 | DC2 (Device Control 2) |
| 13 | 19 | DC3 (XOFF) (Device Control 3) |
| 14 | 20 | DC4 (Device Control 4) |
| 15 | 21 | NAK (Negative Acknowledgment) |
| 16 | 22 | SYN (Synchronous Idle) |
| 17 | 23 | ETB (End of Trans. Block) |
| 18 | 24 | CAN (Cancel) |
| 19 | 25 | EM (End Of Medium) |
| 1a | 26 | SUB (Substitute) |
| 1b | 27 | ESC (Escape) |
| 1c | 28 | FS (File Separator) |
| 1d | 29 | GS (Group Separator) |
| 1e | 30 | RS (Request To Send) |
| 1f | 31 | US (Unit Separator) |
| 20 | 32 | SP (Space) |
| 21 | 33 | ! (Exclamation Mark) |
| 22 | 34 | " (Double Quote) |
| 23 | 35 | # (Number Sign) |
| 24 | 36 | $ (Dollar Sign) |
| 25 | 37 | % (Percent) |
| 26 | 38 | & (Ampersand) |
| 27 | 39 | ` (Single Quote) |
| 28 | 40 | ( (Right / Closing Parenthesis) |
| 29 | 41 | ) (Right / Closing Parenthesis) |
| 2a | 42 | * (Asterisk) |
| 2b | 43 | + (Plus) |
| 2c | 44 | , (Comma) |
| 2d | 45 | - (Minus / Dash) |
| 2e | 46 | . (Dot) |
| 2f | 47 | / (Forward Slash) |
| 30 | 48 | 0 |
| 31 | 49 | 1 |
| 32 | 50 | 2 |
| 33 | 51 | 3 |
| 34 | 52 | 4 |
| 35 | 53 | 5 |
| 36 | 54 | 6 |
| 37 | 55 | 7 |
| 38 | 56 | 8 |
| 39 | 57 | 9 |
| 3a | 58 | : (Colon) |
| 3b | 59 | ; (Semi-colon) |
| 3c | 60 | < (Less Than) |
| 3d | 61 | = (Equal Sign) |
| 3e | 62 | > (Greater Than) |
| 3f | 63 | ? (Question Mark) |
| 40 | 64 | @ (AT Symbol) |
| 41 | 65 | A |
| 42 | 66 | B |
| 43 | 67 | C |
| 44 | 68 | D |
| 45 | 69 | E |
| 46 | 70 | F |
| 47 | 71 | G |
| 48 | 72 | H |
| 49 | 73 | I |
| 4a | 74 | J |
| 4b | 75 | K |
| 4c | 76 | L |
| 4d | 77 | M |
| 4e | 78 | N |
| 4f | 79 | O |
| 50 | 80 | P |
| 51 | 81 | Q |
| 52 | 82 | R |
| 53 | 83 | S |
| 54 | 84 | T |
| 55 | 85 | U |
| 56 | 86 | V |
| 57 | 87 | W |
| 58 | 88 | X |
| 59 | 89 | Y |
| 5a | 90 | Z |
| 5b | 91 | [ (Left / Opening Bracket) |
| 5c | 92 | \ (Back Slash) |
| 5d | 93 | ] (Right / Closing Bracket) |
| 5e | 94 | ^ (Caret / Circumflex) |
| 5f | 95 | _ (Underscore) |
| 60 | 96 | ' (Grave Accent) |
| 61 | 97 | a |
| 62 | 98 | b |
| 63 | 99 | c |
| 64 | 100 | d |
| 65 | 101 | e |
| 66 | 102 | f |
| 67 | 103 | g |
| 68 | 104 | h |
| 69 | 105 | i |
| 6a | 106 | j |
| 6b | 107 | k |
| 6c | 108 | l |
| 6d | 109 | m |
| 6e | 110 | n |
| 6f | 111 | o |
| 70 | 112 | p |
| 71 | 113 | q |
| 72 | 114 | r |
| 73 | 115 | s |
| 74 | 116 | t |
| 75 | 117 | u |
| 76 | 118 | v |
| 77 | 119 | w |
| 78 | 120 | x |
| 79 | 121 | y |
| 7a | 122 | z |
| 7b | 123 | { (Left/ Opening Brace) |
| 7c | 124 | ¦ (Vertical Bar) |
| 7d | 125 | } (Right/Closing Brace) |
| 7e | 126 | ~ (Tilde) |
| 7f | 127 | DEL (Delete) |

- Appendix E：Digit Barcodes

0 ~ 9

0

1

2

3

4

5

6

7

8

9

A – F

A

B

C

D

E

F

- Appendix F：Save/Cancel Barcodes

After scanning the digit barcodes, you need to scan the "Save" barcode to save the data. If you scan the wrong digits, you can cancel the wrong scanned digits.

For example, after scanning a certain setting barcode and digit barcodes "A", "B", "C", "D", you can:

"Delete the Last Digit": The last digit "D" will be deleted.

"Delete All Digits": All digits "ABCD" will be deleted.

"Cancel": All digits "ABCD" will be deleted and the configuration will be cancelled.

Save

Delete the Last Digit

Delete All Digits

Cancel